# Wireshark Network Traffic Analysis

Environment: Localhost (127.0.0.1)

Interface: Loopback Adapter

Server Port: 5000 (Flask)

## 1. Connection Rejection (Service Offline)

This sequence shows the network behavior when the Tkinter client attempts to connect before the Flask server is initialized.

- Packet 215: [SYN] — Client attempt to open a connection.

- Packet 216: [RST, ACK] — Server immediately resets the connection.

- Interpretation: The RST (Reset) flag confirms that no process was listening on Port 5000. This is a crucial data point for demonstrating robust error handling and service availability monitoring.

## 2. TCP Three-Way Handshake (Connection Success)

Once the server was running, the following handshake established a reliable "pipe" for HTTP data.

- Packet 1187: [SYN] — Client synchronization request.

- Packet 1188: [SYN, ACK] — Server acknowledgment and synchronization.

- Packet 1189: [ACK] — Client final acknowledgment.

- Interpretation: This sequence confirms the reliability of the TCP/IP stack on the local loopback, completed in roughly 0.13ms.

## 3. Image List Retrieval (GET Request)

This represents the "Fetch" requirement of the API, where the client requests the current library of images.

- Packet 1190 / 1516: GET /image-list HTTP/1.1

- Packet 1286: HTTP/1.1 200 OK (application/json)

- Interpretation: The server returned a JSON payload containing the filenames. The transition from Packet 1190 to 1286 shows the Application Layer successfully interacting with the OS file system to generate a directory listing.

## 4. Large Image Upload (POST Request)

This is the most technically significant capture, showing the transfer of a large binary file (~343 KB).

- Packet 1817: Len=65495 — Large TCP segment containing binary image data.

- Packet 1818: POST /uploads HTTP/1.1 — The HTTP header defining the destination.

- Packet 1819: Ack=343768 — Acknowledgment of the total data received.

- Interpretation: Because the image exceeded the Maximum Segment Size (MSS), Wireshark performed TCP PDU Reassembly. This triggers the Flask POST route where the Pillow (PIL) library processes the binary stream into a PNG file.

## 5. Resource Management (DELETE Request)

This capture demonstrates the advanced functionality of the API.

- Packet 1545: DELETE /delete-image/myphoto.png HTTP/1.1

- Packet 1547: [PSH, ACK] Len=165

- Interpretation: The use of the DELETE verb (Packet 1545) follows RESTful best practices. The PSH (Push) flag in the response ensures the GUI is updated immediately after the server deletes the file from the /images folder.

## 6. Connection Maintenance (Keep-Alive)

- Packets 1250 / 1251: [TCP Keep-Alive]

- Interpretation: These small packets (45-56 bytes) prevent the connection from timing out. This improves "Technical Delivery" by allowing the client to send multiple requests (like Deleting then Refreshing) without repeating the handshake overhead.